SOC Lab — Phase Checklists

Printable workbook, with commands. Tick as you go.

PROGRESS — through 2026-07-25 Phase A is COMPLETE (VLANs, trunk, pfSense VM + VLAN interfaces, acceptance check passed; snapshot pfsense-clean-install taken). Next up: Phase A.5 (isolation rule). NEW as of 2026-07-29: Phase E now includes the n8n routing layer, and two new phases are woven in — Phase F (Shuffle SOAR) and Phase G (TheHive + Cortex, deferred). Bridge naming built as vmbr2 (NIC2 WAN) / vmbr3 (NIC3 LAN trunk).

## How to use this

Print it, put it next to the keyboard, tick with a pen. Commands are inline so you never flip documents mid-task.

How this actually runs: YOU run the command. YOU paste the output into chat. Claude reads it, tells you whether it worked and why, and gives you the next step. The checklist is the map; Claude is the guide; you are the hands.

Every phase page has the same shape: (1) risk badge, (2) gate — what must be true before you start, (3) steps with commands and → expect lines, (4) green acceptance check, (5) gray explain-it-back.

CLAUDE CANNOT SEE YOUR TERMINAL What you paste is all Claude gets. When something looks wrong, paste the WHOLE output — not a summary. Claude in Chrome CAN see browser tabs (Proxmox, pfSense, Wazuh, Shuffle, n8n, Open WebUI). It cannot see PuTTY, BIOS, iDRAC, or a VM console — copy and paste those.

### Symbols

- → expect: what a correct result looks like. If you don't see it, STOP — don't stack the next change.

- ⚠ a trap — usually one you've already hit, or one that bites people in exactly this spot.

- < > fill in your own value.

COMMANDS GO STALE — VERIFY AGAINST THE LIVE SYSTEM Stable commands (ip, git, docker, Cisco IOS basics) are written out. Anything version- or model-specific — Wazuh/Shuffle tags, Docker package names, switch interface numbering, IOS command availability — check against the live system first. Don't trust a printed version over the actual box.

## The one rule that matters most

Never make a change that could cut off your only access path.

Everything else in this workbook is downstream of that.

# Universal Session Checklist

Run this EVERY time you sit down — before opening any phase page.

### Before you start

☐  Session recording ON — pick the one for where you're working:

# PowerShell (Windows) Start-Transcript -Path "C:\Users\micha\SOC-Lab\Logs\s-$(Get-Date -Format 'yyyy-MM-dd-HHmm').txt"  # Linux (pve01, Kali, Ubuntu) script ~/session.log            # -a to append; exit to stop  # PuTTY (switch) — set BEFORE opening: Session > Logging > All session output

☐  build_log.md open — logging as I go, not reconstructing later

☐  Recovery path confirmed WORKING — tested, not assumed (see Recovery Ladder)

☐  I know what the ONE next change is. Not three.

### During — the loop

☐  Read the step. If I don't understand it, ASK before running it.

☐  Run it.

☐  Paste the whole output into chat — not my summary of it.

☐  Compared ACTUAL output to → expect. Not just 'it didn't error.'

☐  Didn't match? STOPPED. Did not stack another change on top.

☐  Logged it in build_log.md — command, ACTUAL output, decision, why.

### After

☐  Ran the phase's acceptance check — the real one.

☐  Explained it back out loud.

☐  Committed to Git:

git add . && git commit -m "<what changed and why>" && git push

☐  Updated build_log.md — outcome + rollback point.

☐  Stopped the transcript.

# The Recovery Ladder

Tape this to the wall. Work DOWN the list until something works.

| If you lose… | Fall back to… |
| --- | --- |
| Proxmox web UI (192.168.0.201:8006) | SSH to pve01 |
| SSH to pve01 | iDRAC at 192.168.0.100 — or monitor + keyboard on the server |
| iDRAC web console (Java .jnlp pain) | Monitor + keyboard directly on the R710 — always works, skip the Java fight |
| pfSense web UI | Proxmox console into the pfSense VM — or roll back the VM snapshot |
| Cisco switch access | PuTTY over USB-serial console — physical, can't be locked out |
| A VM's network | Check its bridge assignment in the Hardware tab |
| Everything, total mess | Restore VM snapshot / restore pfSense XML config backup |

### Emergency commands — the ones you'll want when it's 11pm

# from the pve01 console (monitor+keyboard, no network needed) ip addr show                    # which bridge has which IP? ip link show                    # UP / DOWN / NO-CARRIER on every NIC ip addr flush dev <bridge>      # rip an IP off a bridge NOW nano /etc/network/interfaces    # the actual config ifreload -a                     # apply without reboot qm list                         # every VM + ID + state qm listsnapshot <vmid>          # what can I roll back to? qm rollback <vmid> <snapname>   # roll it back  # on the switch, via PuTTY serial show running-config show vlan brief show interfaces trunk

### Before ANY high-risk change

☐  Console access tested — right now, not 'last week it worked'

☐  VM snapshot taken:

qm snapshot <vmid> pre-<change>-$(date +%F)

☐  Config backup exported (pfSense XML / switch running-config)

☐  Session recording ON

### Gotchas you have ALREADY hit

- Two bridges claiming the same IP = silent ARP failure, even if one has no cable. Check BOTH with ip addr show.

- Moving a cable/bridge orphans any VM still pointed at the old bridge. Check every VM's Hardware tab after.

- R710 is legacy BIOS only. iDRAC6 config is Ctrl+E during POST, NOT F2. Five-second window.

- A NIC that shows no link may simply never have been brought administratively up (ip link set <iface> up). Check this BEFORE chasing hardware.

- Gi1/0/49 and Gi1/0/50 are hardware-faulty (err-disabled after factory erase) — avoid them.

# Phase A — Network Rebuild

Goal: switch wiped clean, pfSense stood up fresh, three VLANs live.

| RISK: HIGH |
| --- |
| PHASE A COMPLETE — 2026-07-25 All six steps done: switch factory reset, console confirmed, pfSense VM built (CE 2.8.1, VM 102), VLANs 10/20/30 created, Gi1/0/3 trunk scoped to 10,20,30, Gi1/0/48 management access via dedicated USB-adapter link, VLAN interfaces created (10.10.10.1 / .20.1 / .30.1), and the PC on the Management VLAN reached https://10.10.10.1/. Snapshot pfsense-clean-install taken. Full detail is in build_log.md — this page is kept as the record. |  |

### Key commands used (reference)

# switch identification + reset show version show interfaces status write erase reload  # VLANs vlan 10  name MGMT vlan 20  name INFRA vlan 30  name RANGE show vlan brief  # trunk to pfSense (Gi1/0/3) — 2960X rejects encapsulation dot1q, skip it interface Gi1/0/3  switchport mode trunk  switchport trunk allowed vlan 10,20,30 show interfaces trunk  # PC management access port interface Gi1/0/48  switchport mode access  switchport access vlan 10 copy running-config startup-config

EXPLAIN IT BACK What's the difference between a VLAN and a firewall rule — what does each actually control, and why do you need both?

# Phase A.5 — The Isolation Rule

Goal: the Range VLAN is fenced. Nothing out except logs to Wazuh.

RISK: HIGH

### Gate

☐  FRESH pfSense snapshot (not the Phase A one):

qm snapshot <pfsense-vmid> pre-isolation-rule

☐  pfSense XML config exported — Diagnostics > Backup & Restore > Download

☐  Session recording ON

### 1 — WRITE the rule. Do not apply it yet.

pfSense > Firewall > Rules > RANGE tab. Rules evaluate TOP-DOWN, first match wins. Order matters more than anything else on this page.

# Rule 1 — the ONE hole Action: Pass   Interface: RANGE   Protocol: TCP Source: RANGE net   Destination: <Wazuh host IP>   Dest port: 1514 Description: allow range -> wazuh agent ingest ONLY  # Rule 2 — everything else dies here Action: Block   Interface: RANGE   Protocol: Any Source: RANGE net   Destination: Any Description: default deny - range is isolated

⚠ READ IT BACK BEFORE APPLYING Is Pass ABOVE Block? If Block is first, nothing gets through, ever. Does Rule 2 block the interface you manage pfSense FROM? You manage from MGMT (VLAN 10); this rule is on RANGE (VLAN 30) — confirm that's actually true on your screen. This is the #1 self-lockout. 1515 is agent ENROLLMENT — you may need it open temporarily in Phase B, then close it.

### 2 — Apply and test immediately

From a Range VM, all three must fail:

ping 8.8.8.8          # internet      -> expect 100% loss ping 192.168.0.1      # home router   -> expect 100% loss ping <your PC MGMT IP># management    -> expect 100% loss

If ANY of those replies, the fence has a hole. Stop and fix it before Phase C exists.

ACCEPTANCE CHECK From a Range VLAN VM: CANNOT reach internet, home network, or MGMT. (Wazuh port test happens at end of Phase B, once Wazuh exists.)

EXPLAIN IT BACK If Atomic Red Team tried to reach an external IP mid-simulation, what happens under this rule — and why?

# Phase B — Docker / Git / Wazuh

Goal: the substrate — Ubuntu VM, Docker, Git, repo, then Wazuh running with the victim reporting in.

RISK: MEDIUM

### Gate

☐  Phase A.5 acceptance passed — don't build on a broken fence

☐  Session recording ON, build_log.md open

### 1 — Build the Ubuntu VM

⚠ Wazuh wants ~6GB RAM minimum. Give this VM 8GB / 4 cores / 100GB+ disk or the indexer thrashes. Ubuntu Server LTS, Infra VLAN (20), enable OpenSSH during install.

☐  Static IP or DHCP reservation. Write it: ____________________

### 2 — Host prep (indexer needs this or it dies)

sudo sysctl -w vm.max_map_count=262144 echo "vm.max_map_count=262144" | sudo tee -a /etc/sysctl.conf sysctl vm.max_map_count         # -> expect 262144

### 3 — Docker

⚠ Do NOT curl | sh. Use the apt repo method from docs.docker.com/engine/install/ubuntu/ and verify package names live.

docker --version docker compose version          # v2 = space. v1 = docker-compose, hyphen. sudo usermod -aG docker $USER   # then log out/in, or: newgrp docker docker run --rm hello-world     # -> expect "Hello from Docker!"

### 4 — Git + the repo

sudo apt install -y git git config --global user.name "<your name>" git config --global user.email "<your email>"

⚠ Before the first push: anything secret in these files? Credentials, keys, real internal IPs? Check now — Git history is forever, and this repo is your portfolio.

### 5 — Deploy Wazuh

# check the CURRENT tag first — documentation.wazuh.com > Deployment > Docker git clone https://github.com/wazuh/wazuh-docker.git -b <current-tag> cd wazuh-docker/single-node docker compose -f generate-indexer-certs.yml run --rm generator docker compose up -d docker compose ps               # -> expect manager, indexer, dashboard Up

☐  Change the default password. Dashboard: https://<wazuh-host-ip> (port 443)

### 6 — Mount a rules file NOW (needed in Phase C)

# in docker-compose.yml under wazuh.manager > volumes: - ./config/rules/local_rules.xml:/var/ossec/etc/rules/local_rules.xml - ./config/decoders/local_decoder.xml:/var/ossec/etc/decoders/local_decoder.xml

Those host-side files are what you commit — detection-as-code, survives rebuilds.

### 7 — Win11 victim → Range VLAN + Sysmon + agent

☐  Move VM to Range VLAN (Hardware tab). ⚠ CHECK THE HARDWARE TAB AFTER (gotcha #2).

sysmon64.exe -accepteula -i sysmonconfig.xml   # SwiftOnSecurity config Get-Service Sysmon64            # -> expect Running

☐  Install Wazuh agent, point at manager, read Sysmon channel (Microsoft-Windows-Sysmon/Operational, eventchannel)

### 8 — Kali → Range VLAN

☐  Move it. Check its Hardware tab too.

### 9 — Re-test isolation now that Wazuh exists

☐  From Win11 victim: can reach Wazuh on 1514 (must). Still CANNOT reach internet (must not).

ACCEPTANCE CHECK Wazuh dashboard shows Win11-LTSC-victim as ACTIVE, with Sysmon events visible in the stream.

EXPLAIN IT BACK (1) Why is Wazuh three containers on one VM, not three VMs? (2) Why does Git history of a rule — false positive and fix included — beat 'I wrote a detection rule'?

# Phase C — Detection Engineering

Goal: attack it, catch it, then write the rule yourself.

| RISK: MEDIUM |
| --- |
| ART RUNS ON THE VICTIM, NOT KALI Atomic Red Team is a PowerShell test-execution framework, not a remote attack tool. It runs ON the machine you want telemetry from — so it installs on Win11-LTSC-victim. Kali stays your box for real network attacks (nmap, responder), which feed Suricata in Phase C.5. |  |

### Gate

☐  Isolation rule VERIFIED working — you're about to run real attack code

☐  Snapshot Win11 victim — atomics leave the box dirty:

qm snapshot <win11-vmid> pre-atomic-clean

### 1 — Install ART on the victim (PowerShell as admin)

Set-ExecutionPolicy Bypass -Scope Process IEX (IWR 'https://raw.githubusercontent.com/redcanaryco/invoke-atomicredteam/master/install-atomicredteam.ps1' -UseBasicParsing); Install-AtomicRedTeam -getAtomics Get-Command Invoke-Atomic*      # verify the module loaded

⚠ Defender WILL quarantine the atomics folder — that's it working correctly. Add an exclusion for C:\AtomicRedTeam. Safe ONLY because this VM is fenced (Phase A.5).

### 2 — Run your first atomic

Invoke-AtomicTest T1059.001 -ShowDetailsBrief   # look before you leap Invoke-AtomicTest T1059.001 -GetPrereqs Invoke-AtomicTest T1059.001 Invoke-AtomicTest T1059.001 -Cleanup            # always

→ expect:  Wazuh dashboard shows new alerts within ~seconds

### 3 — Write YOUR OWN rule

Claude explains the options and drafts a reference version. YOU write the final rule — this is the skill the job actually pays for.

nano config/rules/local_rules.xml   # user-defined IDs start at 100000 <group name="local,sysmon,">   <rule id="100001" level="10">     <if_group>sysmon_event1</if_group>     <field name="win.eventdata.<field>">…</field>     <description>…</description>     <mitre><id>T1059.001</id></mitre>   </rule> </group> docker compose restart wazuh.manager

### 4 — Prove it, then commit it

☐  Re-run the SAME atomic. Does YOUR rule fire (ID 100001+)?

git add config/rules/local_rules.xml git commit -m "detect: T1059.001 powershell exec - custom rule 100001"

ACCEPTANCE CHECK A committed rule of YOUR authorship fires correctly on a re-run of the same technique.

EXPLAIN IT BACK What's the difference between a log and an alert — and what has to happen for one to become the other?

# Phase C.5 — Network Visibility (SPAN + Suricata)

Goal: verify the tap, mirror the Range VLAN, correlate both layers.

RISK: HIGH

### 1 — Verify the USB adapter BEFORE building on it

lsusb                 # ASIX AX88179 / Realtek RTL8153 = well supported ip link show          # new enx* interface appear? iperf3 -s / iperf3 -c <box>   # cheap USB2 adapters drop packets under load

☐  DECISION: adapter is good  /  falling back to NIC4. Why: ____________________

⚠ One SPAN adapter is currently borrowed for PC management access — free it or confirm the second unit first.

### 2 — SPAN session on the switch

monitor session 1 source vlan 30 monitor session 1 destination interface <monitor port> show monitor session 1        # -> expect Source VLANs 30, Destination = your port

⚠ A SPAN destination port stops being a normal port. Don't pick one you use for anything else.

### 3 — Wire it up (tap gets NO IP)

☐  Pass the adapter through to the WAZUH host — not Kali (Kali was only the compatibility stand-in).

sudo ip link set <iface> up sudo ip link set <iface> promisc on ip addr show <iface>          # -> MUST show no inet address

### 4 — Suricata

sudo apt install -y suricata sudo suricata-update sudo nano /etc/suricata/suricata.yaml     # point af-packet at the tap sudo suricata -T -c /etc/suricata/suricata.yaml   # config test FIRST sudo systemctl restart suricata sudo tail -f /var/log/suricata/eve.json

☐  Point Wazuh at eve.json (ossec.conf, <log_format>json</log_format>)

### 5 — Prove both layers see the same event

# from Kali — a real network attack, not an atomic: nmap -sS -p- <win11-victim-ip>

ACCEPTANCE CHECK Suricata flags the scan in Wazuh, AND a host-based alert exists for the same event. Two layers, one story.

EXPLAIN IT BACK If an attacker hit a device with no Sysmon and no agent at all, how could Suricata still catch it?

# Phase D — AI Triage + Agent-Identity Governance

Goal: AI drafts the verdict. You decide. The agent is governed like any identity.

RISK: MEDIUM

### 1 — Identity FIRST, service second

Order is deliberate: build the service first and you'll wire in your own credential 'just for now' and never revisit it. That's how orgs end up with ungoverned agents.

☐  Create a dedicated Wazuh API user for the agent — NOT your admin login

☐  Role + policy scoped read-only: agent:read, alerts:read — NOT delete, NOT admin

# token as the agent user: curl -k -u <agent-user>:<pass> -X POST "https://<wazuh>:55000/security/user/authenticate?raw=true" # should WORK (read): curl -k -H "Authorization: Bearer <tok>" "https://<wazuh>:55000/agents?pretty" # should FAIL 403 (delete) — that IS the test: curl -k -H "Authorization: Bearer <tok>" -X DELETE "https://<wazuh>:55000/agents?pretty"

→ expect:  read = 200 OK. delete = 403 Forbidden. If delete succeeds, your scope is wrong — fix before continuing.

### 2 — agent-registry.md

☐  Create it in the repo root. Entry for wazuh-triage-01: purpose, may access (alerts:read, agent:read), may NOT (delete/modify rules/leave Infra pipeline/act on its own verdict), owner (Michael), credential type + rotation, status, audit trail, last review.

### 3 — Separate the audit trail

☐  Agent actions log agent=wazuh-triage-01; your reviews log user=michael. Read one log line — can you tell who acted? If ambiguous, fix it now.

### 4 — Build the service

☐  On the Infra host: poll Wazuh alerts → model → verdict suggestion, under the AGENT's credential, never your session

☐  You review and confirm/override. Logged separately.

ACCEPTANCE CHECK An alert flows end to end: detection → Wazuh alert → agent (own identity) suggests a verdict → your reviewed verdict. Both logged distinguishably. Registry matches reality.

EXPLAIN IT BACK (1) Why does an AI verdict need a human verdict logged beside it? (2) Why is the agent using your personal login a governance gap, in least-privilege and audit terms?

# Phase E — Local AI + n8n Routing

Goal: route cheap triage local, escalate the hard stuff — and give yourself a local-first personal AI.

| RISK: LOW |
| --- |
| THIS PHASE DOES NOT BUILD THE OLLAMA STACK Ollama itself belongs to the ai-node project (C:\Users\micha\lab\ai-node\hybrid_ai_node_build_plan.md, its Phase 6). Check THAT project's build_log.md for status before starting. This page is integration + routing only. |  |

### Gate

☐  ai-node Phase 6 complete — verified in ITS build_log.md, not assumed

☐  Infra → ai-vm route exists and pfSense allows it

### 1 — Confirm the node is actually reachable

# from the Wazuh/Infra host: ping 192.168.0.203 curl http://192.168.0.203:11434/api/tags     # Ollama's model list curl http://192.168.0.203:11434/api/generate -d '{"model":"<m>","prompt":"say ok","stream":false}'

→ expect:  JSON listing your pulled models, then a JSON reply from the model

### 2 — Open WebUI (your personal chat UI)

☐  Stand up Open WebUI as a service container on pve01, pointed at Ollama

☐  Local models as default; Claude / Grok selectable in the model dropdown

### 3 — n8n (the router)

☐  Stand up n8n as a service container on pve01

☐  Simple / low-severity → local model on ai-vm

☐  Ambiguous / high-severity → escalate to claude -p (subscription, no API billing) or Grok

☐  Write your routing threshold — what makes something escalate? (two-tier rule: Claude drafts a reference, YOU write the final)

⚠ Two engines, two lanes: n8n = AI routing + general automation, Shuffle = SOC response (Phase F). Never build both engines in the same stretch.

### 4 — Register it — it's a second agent

☐  Add triage-router-01 to agent-registry.md as its OWN entry — it makes autonomous escalate/local decisions, so it's governed regardless of which engine hosts the logic

## agent: triage-router-01 purpose:      routes alerts local vs escalate may access:   Ollama API on ai-vm; claude -p on the Infra host may NOT:      touch the Wazuh API; write verdicts directly owner:        Michael Mathews credential:   ____________________   rotate: ____________ status:       active      last review: ____-__-__

ACCEPTANCE CHECK One full triage cycle completes on the local model with NO external API call; a second escalates correctly. Registry entry accurate.

EXPLAIN IT BACK Give one alert you'd trust the local model to handle alone, and one you'd escalate — and what makes the difference.

# Phase F — SOAR (Shuffle)

Goal: automate the response — alert to enriched case in seconds, not an hour.

| RISK: MEDIUM |
| --- |
| WHY THIS PHASE EXISTS SOAR is named directly on the SOC job listings you're targeting. Shuffle is the community-standard open-source SOAR engine and pairs natively with Wazuh. Buildable as soon as Phase C is done (you need real detections to respond to) — it does not depend on D or E. |  |

### Gate

☐  Phase C acceptance passed — you have detections worth automating

☐  Infra outbound internet rule written + reviewed (enrichment needs it — two-tier rule)

☐  Session recording ON, build_log.md open

### 1 — Build a dedicated Shuffle VM

⚠ Own VM on the Infra VLAN. Do NOT co-locate with Wazuh — both are memory-hungry. Docker deploy; verify the current image/tag against Shuffle's docs, not from memory.

☐  Ubuntu Server, Infra VLAN (20), sized for a Docker service. Write the IP: ____________________

### 2 — Wire Wazuh → Shuffle

Add an integration block to Wazuh's config so alerts POST to a Shuffle workflow. Note the rule_id you scope it to.

<integration>   <name>shuffle</name>   <hook_url>https://<shuffle-host>/api/v1/hooks/<id></hook_url>   <rule_id>100001</rule_id>   <alert_format>json</alert_format> </integration> docker compose restart wazuh.manager

### 3 — Build a first playbook

You write the final workflow logic; Claude drafts a reference (two-tier rule). Flow: alert in → parse IOCs → light enrichment (VirusTotal / AbuseIPDB via HTTP) → notify.

☐  Webhook node receives the Wazuh alert

☐  Parse the IOC (e.g. SHA256 capture group) — test against a REAL alert payload, not the placeholder dummy text

☐  HTTP node queries VirusTotal / AbuseIPDB with your API key

☐  Notify (email / Discord / back into Wazuh)

### 4 — Commit it

git add shuffle/  && git commit -m "soar: T1059.001 enrich + notify playbook"

⚠ OPTIONAL — ACTIVE RESPONSE IS GATED A block-IP-via-pfSense step opens a NEW path into Range/pfSense. That's a firewall decision written and reviewed under the two-tier rule. Do NOT build auto-response until you've explicitly decided you want it. Enrichment-only playbooks need no inbound path.

ACCEPTANCE CHECK A real detection fires → Shuffle receives it → enriches the IOCs → produces a case/notification end to end. Manual alert-to-case is 35–60 min; automated is seconds. That before/after is the interview artifact.

EXPLAIN IT BACK Why is 'Shuffle enriched the IOCs and opened the case in 90 seconds' a stronger interview line than 'I set up a SOAR tool'? And why does the router (n8n) get an agent-registry entry but Shuffle does not?

# Phase G — Case Management (TheHive + Cortex)

Goal (LATER): a real analyst case desk + systematic enrichment. Deferred on purpose.

| RISK: HIGH EFFORT |
| --- |
| DEFERRED — DO NOT START UNTIL F AND E ARE STABLE TheHive and Cortex are one matched-pair unit (both from StrangeBee). Add them together, later, against a calm backdrop — not stacked on top of an in-progress build. |  |

| App | Job |
| --- | --- |
| TheHive | Case management — tickets, timeline, tasks, observables, status. The analyst's desk. No overlap with anything else in the stack. |
| Cortex | Enrichment/analysis engine — runs analyzers against IOCs at scale. Overlaps with Shuffle's light enrichment; Cortex is the systematic version. |
| WHY IT'S DEFERRED — THE DEPENDENCY HELL TheHive stores no data itself. It requires Cassandra (case DB) + Elasticsearch (search) as backends — all three healthy and version-matched at once. Its Elasticsearch must coexist with Wazuh's own indexer. Realistic cost with friction: ~4–8 sessions, mostly version-matching, startup-order, and memory issues — not conceptual difficulty. Give it its own dedicated VM. |  |

### When you do start (outline only)

- Dedicated TheHive VM on Infra. Stand up Cassandra + Elasticsearch first; confirm both healthy before TheHive.

- Match versions deliberately — TheHive is picky about which Cassandra/Elasticsearch it accepts. Check the live docs.

- Stand up Cortex, add analyzers (VirusTotal, AbuseIPDB, etc.), connect it to TheHive.

- Point Shuffle's playbook at TheHive so cases get created automatically instead of just notified.

ACCEPTANCE CHECK A Shuffle-enriched alert auto-creates a TheHive case with observables attached; a Cortex analyzer runs against one observable and returns a verdict.

EXPLAIN IT BACK What does TheHive give you that Shuffle alone doesn't — and why is case management its own distinct skill from detection or automation?

# Investigation Writeup

Print several. One per investigation. Half a page is plenty — these ARE the portfolio.

Date: ______________     Alert / technique: __________________________________________

### What happened (plain language)

____________________________________________________________________________________

### What I saw (the actual alert / log evidence)

____________________________________________________________________________________

### My verdict — circle one:   MALICIOUS    BENIGN    NEEDS MORE INFO

Why: _______________________________________________________________________________

### What I'd do next (contain / escalate / tune the rule / ignore)

____________________________________________________________________________________

### If an AI verdict was involved — did I agree? If not, who was right?

____________________________________________________________________________________

Committed as  investigations/____-__-__-______________________.md   ☐

# Checkpoint Tracker

Tick ONLY when you can explain it out loud, cold, without notes. That's the bar.

| Mod | Phase | Can I explain… | Yes |
| --- | --- | --- | --- |
| 0 | pre-A | Why Kali lost network after the NIC1/NIC2 cable swap (bridge = wall socket) | ☐ |
| 1 | A | VLAN vs firewall rule — what each controls, why you need both | ☐ |
| 2 | A.5 | Default-deny vs block-known-bad — what happens when ART reaches for the internet | ☐ |
| 3 | B | Containers vs VMs — why Wazuh is 3 containers on 1 VM | ☐ |
| 4 | B | Why Git history beats 'I wrote a rule' in an interview | ☐ |
| 5 | C | Log vs alert — what turns one into the other | ☐ |
| 6 | C.5 | How Suricata catches an agentless device; why a SPAN port has no IP | ☐ |
| 7 | D | Why an AI verdict needs a human verdict logged beside it | ☐ |
| 7.5 | D | Why the agent using your personal login is a governance gap | ☐ |
| 8 | E | Which alerts stay local vs escalate — and what makes the difference | ☐ |
| 9 | F | Why SOAR turns a 40-minute manual triage into a 90-second automated one | ☐ |
| 10 | F | Why n8n gets an agent-registry entry but Shuffle doesn't | ☐ |
| 11 | G | What case management (TheHive) adds beyond detection and automation | ☐ |

## The final test — for ANY phase, out loud:

- What did I build?

- What problem does it solve?

- What went wrong while building it?

- How did I fix it?

Question 3 is the one interviewers lean on hardest. Don't smooth it over: the pfSense lockout, the duplicate-IP ARP failure, the orphaned bridge — those are the answers that sound like someone who has actually done the work.